--- System Configuration Dialog ---

Would you like to enter the initial configuration dialog? [yes/no]: no

Press RETURN to get started!

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#hostname A

A(config)#interface fa1/0

A(config-if)#ip address 10.0.0.3 255.0.0.0

A(config-if)#no shutdown

A(config-if)#

%LINK-5-CHANGED: Interface FastEthernet1/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet1/0, changed state to up

A(config)#interface fa0/0

A(config-if)#ip address 172.16.0.1 255.255.0.0

A(config-if)#no shutdown

A(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

A(config-if)#interface fa0/1

A(config-if)#ip address 192.168.0.1 255.255.255.0

A(config-if)#no shutdown

A(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to up

A(config-if)#exit

A(config)#ip route 11.0.0.0 255.0.0.0 172.16.0.2

A(config)#

--- System Configuration Dialog ---

Would you like to enter the initial configuration dialog? [yes/no]: no

Press RETURN to get started!

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#hostname B

B(config)#

B(config)#

B(config)#

B(config)#interface fa0/0

B(config-if)#ip address 172.16.0.2 255.255.0.0

B(config-if)#no shutdown

B(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0, changed state to up

B(config-if)#

B(config-if)#

B(config-if)#interface fa1/0

B(config-if)#ip address 11.0.0.3 255.0.0.0

B(config-if)#no shutdown

B(config-if)#

%LINK-5-CHANGED: Interface FastEthernet1/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet1/0, changed state to up

B(config)#interface fa0/1

B(config-if)#ip address 193.168.0.1 255.255.255.0

B(config-if)#no shutdown

B(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to up

B(config-if)#exit

B(config)#ip route 10.0.0.0 255.0.0.0 172.16.0.1

B(config)#

--- System Configuration Dialog ---

Would you like to enter the initial configuration dialog? [yes/no]: no

Press RETURN to get started!

Router>

Router>

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#hostname C

C(config)#interface fa0/0

C(config-if)#ip address 192.168.0.2 255.255.255.0

C(config-if)#no shutdown

C(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0, changed state to up

C(config-if)#interface fa0/1

C(config-if)#ip address 193.168.0.2 255.255.255.0

C(config-if)#no shutdown

C(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up